#include <Servo.h> //library untuk servo

#include <LiquidCrystal.h> //library untuk LCD

const int trig = 2, echo = 3;

int z;

int red=A0,orange=A1,yellow=A2,green=A3; //mendeklarasikan variabel dengan nama Red, orange, Yellow, dan Green dengan pin A0, A1, A2, A3

int buzzer=7;

int duration, Distanceincm;

Servo myservo; // variable to store the data position

LiquidCrystal lcd(13, 12, 11, 10, 9, 8);

void setup() {

lcd.begin(16, 2); //16 = Column, 2 = Line

pinMode(trig, OUTPUT); //set the mode of the trig as OUTPUT

pinMode(echo, INPUT); //set the mode of the echo as INPUT

pinMode(red, OUTPUT); //set the mode from red as OUTPUT

pinMode(yellow, OUTPUT); //set the mode from yellow as OUTPUT

pinMode(green, OUTPUT); //set the mode from green as OUTPUT

pinMode(orange, OUTPUT); //set the mode from orange as OUTPUT

digitalWrite(green, HIGH); //makes the green variable value HIGH

myservo.attach(4); //The servo motor cable data signal is connected to pin 4 of the Arduino

}

void loop() {

digitalWrite(trig, HIGH); //make the value of the trig variable HIGH

delay(15); //time delay of 15 ms

digitalWrite(trig, LOW); //make the trig variable value LOW

duration = pulseIn(echo, HIGH); //pulseIn totals the waiting time of echoPIn when it is high, the waiting time will be stored in memory

Distanceincm = (duration / 58.2); //calculations to be used as the distance from the ultrasonic to the surface of the water

z=335-Distanceincm; //calculation of water level

lcd.setCursor(0, 0); //set at column 0 and row 0

lcd.print("Height : "); //write down "Height:"

lcd.print(z); //write down the calculation results of the variable z

delay(1000); //time delay of 1 second

if (Distanceincm > 240) {

lcd.clear(); //to clear the LCD screen

lcd.setCursor(0, 1); //set on column 0 and row 1

lcd.print("Condition: Safe "); //write down "Condition: Safe"

digitalWrite(red, LOW);

digitalWrite(orange, LOW);

digitalWrite(yellow, LOW);

digitalWrite(green, HIGH);

noTone(buzzer); //the buzzer does not sound

myservo.write(0); //move the servo by 0 degrees

delay(20); //time delay of 20 ms

}

if (Distanceincm > 160 && Distanceincm <= 240 ) {

lcd.setCursor(0, 1); //set on column 0 and row 1

lcd.print("Low"); //writes "Condition: Standby 1"

digitalWrite(red, LOW);

digitalWrite(orange, LOW);

digitalWrite(yellow, HIGH);

digitalWrite(green, LOW);

noTone(buzzer); //the buzzer does not sound

myservo.write(30); //move the servo by 30 degrees

delay(20); //time delay of 20 ms

}

if (Distanceincm > 80 && Distanceincm <= 160) {

lcd.setCursor(0, 1);

lcd.print("Medium"); //writes "Condition: Standby 2"

digitalWrite(red, LOW);

digitalWrite(orange,HIGH);

digitalWrite(yellow, LOW);

digitalWrite(green, LOW);

noTone(buzzer);

myservo.write(60); //move the servo by 60 degrees

delay(20); //time delay of 20 ms

}

if (Distanceincm > 0 && Distanceincm <= 80) {

lcd.setCursor(0, 1); //set on column 0 and row 1

lcd.print("DANGER!!"); //write down "Condition: DANGER!"

digitalWrite(red, HIGH);

digitalWrite(orange,LOW);

digitalWrite(yellow, LOW);

digitalWrite(green, LOW);

buzz(); //go to label / buzz function to sound the buzzer

myservo.write(90); //move the servo by 90 degrees

delay(20); //time delay of 20 ms

}

}

void buzz(){

tone(buzzer,2000,250); //buzzer sound setting with a frequency of 2000 Hz and a long time of 250 ms

delay(300); //time delay of 300 ms

}